Robot Project Notes:

Phase 1:

Dates: 9/1 – 9/29

BOM:

| Description | Part No. | Qty |
| --- | --- | --- |
| WWZMDiB L298N Motor Driver Controller Board DC Dual H Bridge Module | B0CR6BX5QL | 1 |
| DC12V Encoder Gear Motor with Mounting Bracket, 65mm Magnetic Motor | B07X7M1LLQ | 2 |
| Arduino UNO R4 WiFi [ABX00087] – Renesas RA4M1 + ESP32-S3, Wi-Fi, Bluetooth, USB-C, CAN, 12-bit DAC, OP AMP, Qwiic Connector, 12x8 LED Matrix | B0C8V88Z9D | 1 |
| Chassis | 3D printed | 1 |
| Wires | Generic | As needed |
| Breadboard |  | 1 |
| 12V DC Wall Wart | Generic | 1 |

Chassis used:

EricBot v3 model (first print)

EricBot v6 model (second printed version)

Robot Picture:

Revision 1: Mounted Wheels, fit check some components.

Revision 2: Mounted components and wiring

Schematic:

Code notes:

- VSCode IDE

- Connection to Arduino facilitated by PlatformIO

- Manually write encoder code, must use ISR to function well (polling doesn’t work well)

- Encapsulating ISR’s in Classes using static functions and pointers

- Drive motor using PWM on enaPins on L298N, direction defined by 2 input pins per motor

- Github: https://github.com/eriico39hi/EricBot

Goals for this phase:

- Read motor encoder in Arduino console - DONE

- Drive motors through L298N driver PCB’s

References:

- https://www.reddit.com/r/arduino/comments/ll5avx/is_anyone_using_visual_studio_code_for_arduino/

- https://dronebotworkshop.com/platformio/

- https://howtomechatronics.com/tutorials/arduino/arduino-dc-motor-control-tutorial-l298n-pwm-h-bridge/

- https://forum.arduino.cc/t/using-a-class-member-for-isr-is-it-possible/380127

- https://howtomechatronics.com/tutorials/arduino/rotary-encoder-works-use-arduino/

Phase 2:

9/29 – TBD

Code notes:

- ROS2 and Ubuntu Raspberry Pi install

- VNC configuration?

Goals for this phase:

- Communicate Arduino to ROS

- Create Kinematics to define robot in terms on m and deg

- PID tuning

Phase 3:

November?

Goals for this phase:

- Motion Program

- Write report